Tax Deduction Qualified Software

DOE-2.2 version 47d

On this page you'll find information about DOE-2.2 version 47d qualified computer software (buildings.energy.gov/qualified_software.html), which calculates energy and power cost savings that meet federal tax incentive requirements for commercial buildings.

Date Documentation Received by DOE: 24 September 2009

Statements in quotes are from the software developer.

| Internal Revenue Code §179D (c)(1) and (d) Regulations Notice 2006-52, Section 6 requirements as amplified by Notice 2008-40, Section 4 requirements. |  |
| --- | --- |
| (1) The name, address, and (if applicable) web site of the software developer; | James J. Hirsch & Associates 12185 Presilla Road Camarillo, California 93012-9243 www.doe2.com |
| (2) The name, email address, and telephone number of the person to contact for further information regarding the software; | Jeff Hirsch James J. Hirsch & Associates Jeff.Hirsch@DOE2.com 805-553-9000 (phone) 805-532-2401 (fax) |
| (3) The name, version, or other identifier of the software as it will appear on the list; | DOE-2.2 v47d |
| (4) All test results, input files, output files, weather data, modeler reports, and the executable version of the software with which the tests were conducted; and | Provided to DOE. |
| (5) A declaration by the developer of the software, made under penalties of perjury, that— |  |
| (a) The software has been tested according to ANSI/ASHRAE Standard 140-2007 Standard Method of Test for the Evaluation of Building Energy Analysis Computer Programs; | "The software has been tested according to ANSI/ASHRAE Standard 140-2007 Standard Method of Test for the Evaluation of Building Energy Analysis Computer Programs." |
| (b) The software can model explicitly— |  |
| (i) 8,760 hours per year; | "The DOE-2.2 v47d software complies." |
| (ii) Calculation methodologies for the building components being modeled; | "The DOE-2.2 v47d software complies." |
| (iii) Hourly variations in occupancy, lighting power, miscellaneous equipment power, thermostat setpoints, and HVAC system operation, defined separately for each day of the week and holidays; | "The DOE-2.2 v47d software complies." |
| (iv) Thermal mass effects; | "The DOE-2.2 v47d software complies." |
| (v) Ten or more thermal zones; | "The DOE-2.2 v47d software complies." |
| (vi) Part-load performance curves for mechanical equipment; | "The DOE-2.2 v47d software complies." |
| (vii) Capacity and efficiency correction curves for mechanical heating and cooling equipment; and | "The DOE-2.2 v47d software complies." |
| (viii) Air-side and water-side economizers with integrated control. | “The DOE-2.2 v47d software complies with the air-side economizer requirements and with two forms of water-side economizers (WSE): dedicated WSE coils in air handlers and ‘parallel’ (i.e., non-integrated) WSE such as a strainer cycle. The DOE-2.2 v47d software cannot model ‘parallel’ WSE (i.e., with integrated control) and shall not be used for projects with that technology." |
| (c) The software can explicitly model each of the following HVAC systems listed in Appendix G of Standard 90.1-2004: |  |
| (i) Packaged Terminal Air Conditioner (PTAC) (air source), single-zone package (through the wall), multi-zone hydronic loop, air-toair DX coil cooling, central boiler, hot water coil. | "The DOE-2.2 v47d software complies." |
| (ii) Packaged Terminal Heat Pump (PTHP) (air source), single-zone package (through the wall), air-to-air DX coil heat/cool. | "The DOE-2.2 v47d software complies." |
| (iii) Packaged Single Zone Air Conditioner (PSZ-AC), single-zone air, air-to-air DX coil cool, gas coil, constant-speed fan. | "The DOE-2.2 v47d software complies." |
| (iv) Packaged Single Zone Heat Pump (PSZ-HP), single-zone air, air-to-air DX coil cool/heat, constant-speed fan. | "The DOE-2.2 v47d software complies." |
| (v) Packaged Variable-Air-Volume (PVAV) with reheat, multi-zone hydronic loop, air-to-air DX coil, VAV fan, boiler, hot water VAV terminal boxes. | "The DOE-2.2 v47d software complies." |
| (vi) Packaged Variable-Air-Volume with parallel fan powered boxes (PVAV with PFP boxes), multi-zone air, DX coil, VAV fan, fan-powered induction boxes, electric reheat. | "The DOE-2.2 v47d software complies." |
| (vii) Variable-Air-Volume (VAV) with reheat, multi-zone air; multi-zone hydronic loop, air-handling unit, chilled water coil, hot water coil, VAV fan, chiller, boiler, hot water VAV boxes. | "The DOE-2.2 v47d software complies." |
| (viii) Variable-Air-Volume with parallel fan powered boxes (VAV with PFP boxes), multi-zone air, air-handling unit, chilled water coil, hot water coil, VAV fan, chiller, fan-powered induction boxes, electric reheat. | "The DOE-2.2 v47d software complies." |
| (d) The software can— |  |
| (i) Either directly determine energy and power costs or produce hourly reports of energy use by energy source suitable for determining energy and power costs separately; and | "The DOE-2.2 v47d software complies." |
| (ii) Design load calculations to determine required HVAC equipment capacities and air and water flow rates. | "The DOE-2.2 v47d software complies." |
| (e) The software can explicitly model: |  |
| (i) Natural ventilation. | "The DOE-2.2 v47d software can model simple single-zone natural ventilation using air changes per hour (user-defined) or Sherman-Grimsrud (calculated)." |
| (ii) Mixed mode (natural and mechanical) ventilation. | "The DOE-2.2 v47d software does not explicitly model this feature and shall not be used for projects with that technology." |
| (iii) Earth tempering of outdoor air. | "The DOE-2.2 v47d software does not explicitly model this feature and shall not be used for projects with that technology." |
| (iv) Displacement ventilation. | "The DOE-2.2 v47d software does not explicitly model this feature and shall not be used for projects with that technology." |
| (v) Evaporative cooling. | "The DOE-2.2 v47d software complies." |
| (vi) Water use by occupants for cooking, cleaning or other domestic uses. | "The DOE-2.2 v47d software does not explicitly model this feature and shall not be used for projects with that technology." |
| (vii) Water use by heating, cooling, or other equipment, or for on-site landscaping. | "The DOE-2.2 v47d software does not explicitly model this feature and shall not be used for projects with that technology." |
| (viii) Automatic interior or exterior lighting controls (such as occupancy, photocells, or time-clocks). | "The DOE-2.2 v47d software can explicitly model automatic interior or exterior lighting controls such as occupancy sensors or time-clocks, but cannot model photocells." |
| (ix) Daylighting (sidelighting, skylights, or tubular daylight devices). | "The DOE-2.2 v47d software complies." |
| (x) Improved fan system efficiency through static pressure reset. | "The DOE-2.2 v47d software does not explicitly model this feature and shall not be used for projects with that technology." |
| (xi) Radiant heating or cooling (low or high temperature). | "The DOE-2.2 v47d software complies with the radiant system requirements for low delta-t heating panel applications. The DOE-2.2 v47d software does not explicitly model radiant cooling systems or high temperature radiant heating systems and shall not be used for projects with that technology." |
| (xii) Multiple or variable-speed control for fans, cooling equipment, or cooling towers. | "The DOE-2.2 v47d software complies." |
| (xiii) On-site energy systems (such as combined heat and power systems, fuel cells, solar photovoltaic, solar thermal, or wind). | "The DOE-2.2 v47d software can model on-site energy systems including engines, gas turbines, steam turbine generators and photovoltaic arrays. DOE-2.2 v47d cannot model fuel cells, solar thermal, or wind systems and shall not be used for projects with these technologies." |